# 09 6816 	CARPET SYSTEMS

FINAL:  	Feb. 12, 2009

Note:  The following specification criteria have been assembled to promote the procurement of reduced-emission nylon face-fiber carpet systems. Additional considerations related to product toxicity or other High Performance building criteria are included where appropriate.

PART 1 - GENERAL

1.01  SUMMARY

- This section includes commercial nylon face-fiber carpet systems which meet specified criteria for:

- Emissions testing for improved indoor air quality; and

- Reduced-emission, reduced-toxicity installation adhesives.

  - REFERENCES / QUALITY ASSURANCE

(for indoor air quality and toxicity criteria)

- Adhesives (VOC content):

South Coast Air Quality Management District Rule #1168, Adhesive and Sealant Applications, amended 9/15/2000, South Coast Air Quality Management District (SCAQMD), State of California, www.aqmd.gov.

- Adhesives (prohibited compounds):

“Green Seal Environmental Standard for Certification of Commercial Adhesives” (GS-36),

Green Seal, Washington, DC, www.greenseal.org

- Emissions testing:

“Green Label” program for emissions testing and emission standards for carpet, carpet adhesives, and carpet cushion. Carpet and Rug Institute, Dalton, GA, www.carpet-rug.com

- Installation:

“Standard for Installation of Commercial Carpet”, CRI 104, Carpet and Rug Institute,

Dalton, GA, www.carpet-rug.com

1.03	RELATED DOCUMENTS

Waste Prevention Specifications, Section 09690 (Carpet Tiles), as included in the New York City Department of Design and Construction Design Guide for Consultants.

1.04 SUBMISSION REQUIREMENTS

A.	Manufacturer’s certification of product compliance with adhesive standards (VOC content and prohibited compounds) per section 2.01 of this specification.

- Manufacturer’s certification of product compliance with emissions testing standards per section 2.01 of this specification.

- Material Safety Data Sheets.

- Manufacturer’s maintenance and cleaning instructions.

1.05	DELIVERY AND STORAGE

- Where feasible, remove carpet or carpet tiles from packaging and store in unoccupied, ventilated areas (100% outside air supply, minimum of 1.5 air changes per hour, no recirculation) for 24-72 hours prior to installation. Carpet or carpet tiles shall not be stored with materials which have high emissions of VOCs or other contaminants (see section 3.01 below).

- Carpets or carpet tiles to be stored per manufacturer’s recommendations for allowable temperature and humidity range. Carpets or carpet tiles shall not be allowed to become damp.

PART 2 – PRODUCTS

2.01 MATERIALS

- Environmentally-Preferable Product Criteria:

- Emissions Testing:
Carpets, carpet tiles, carpet pads, and adhesives shall meet or surpass all criteria of the “Green Label” Indoor Air Quality Test Program established by the Carpet and Rug Institute (CRI) of Dalton, Georgia.

2.	VOC content of Adhesives:

The volatile organic compound (VOC) content of carpet and carpet pad adhesives shall not exceed the limits defined in Rule #1168, “Adhesive and Sealant Applications” of the South Coast Air Quality Management District (SCAQMD), of the State of California. The VOC limits defined by SCAQMD (based on 9/15/00 amendments) are as follows. All VOC limits are defined in grams per liter, less water and less exempt compounds (determined by U. S. EPA Reference Test Method 24).

- Indoor carpet adhesive		50

- Carpet pad adhesive		50

3. 	Prohibited Compounds in Adhesives:

To the extent feasible, carpet adhesives shall comply with the prohibited compound requirements of the “Green Seal Environmental Standard for Certification of Commercial Adhesives” (GS-36), Green Seal, Washington, DC. Prohibited compounds, as defined in the Green Seal standard, shall include:

- Carcinogens;

- Reproductive Toxins;

- Persistent, Bioaccumulative and Toxic Compounds (PBTs); and

- Ozone-depleting Substances (ODSs).

PART 3 –  EXECUTION

3.01	INSTALLATION

- Comply, at minimum, with CRI 104 recommendations for space ventilation during and after carpet system installation. Where feasible, the following ventilation conditions shall be maintained during carpet system installation, and for a period of 72 hours after installation: 1) supply 100% outside air 24 hours a day; 2) supply airflow at a rate of 6 air changes per hour, when outside temperatures are between 55 degrees F and 85 degrees F and humidity is between 30% and 60%; and 3) supply airflow at a rate of 1.5 air changes per hour, when outside air conditions are not within the range stipulated in item 2 above.

- Where feasible, carpet systems shall be installed after the installation of materials or finishes which have high short-term emissions of VOC’s, formaldehyde, particulates, or other air-borne compounds which may be adsorbed by or settle on the carpet tiles. Materials with high short-term emissions include, but are not limited to: adhesives, sealants and glazing compounds (specifically those with petrochemical vehicles or carriers); paint, wood preservatives, and finishes; control and/or expansion joint fillers; hard finishes requiring adhesive installation; gypsum board (with associated finish processes and products); and composite or engineered wood products with formaldehyde binders.

- When installed over concrete slabs, verify compatibility of carpet tile and adhesives with curing compounds, leveling agents, or sealing agents used for slab preparation.

END OF SECTION 09680